JPNIC Translated Document

Source document: jpnic/jp-named.doc

Date of the source: October 1, 1991

Date of the last update of this translation: February 14, 1996

This is a translation of a JPNIC document. JPNIC provides this

translation for convenience of those who can not read Japanese. But it

may contain mis-translations, and is by no means official. One should

consult the source document written in Japanese for detail.

-----------------------------------------------------------------------

ON NAME SERVERS AND THEIR SETTING (Ver. 1.7)

October 3, 1989

(Final revision date: October 1, 1991)

Hiroaki Takada

Information Sciences Division, Faculty of Science,

University of Tokyo, Japan

hiro@is.s.u-tokyo.ac.jp

NOTE: Since this document was written at the initial period of

starting up domestic name servers, the readers are requested to

understand that there will be many points in this document that

do not agree with the present situation.  Also, any volunteers

willing to fully update this document would be welcomed heartily.

This document is prepared centered around the technical descriptions

of the information required at the time of setting name servers.

The method of acquiring an IP address, the method of acquiring a

domain name, obtaining permission of connection to a domestic IP

network, and obtaining permission of connection to Internet are

all outside the scope of this document.

1. What is a Name Server

In general, a name server is a server used for searching an IP

address from a host name.  Although there are a number of standards

for name servers, the standard used in Internet, that is, BIND

(Berkeley Internet Name Domain Server) is described below.  For more

details of BIND, the reader should consult the documents given in

the reference.

The feature of BIND is that it has a structure of a distributed

database supporting domain names with a hierarchial structure.  The

management of the addresses within the respective domains is carried

out by the manager of that domain.

Further, a name server plays a very important role at the time of

transferring electronic mail.  The mail transferring rule being used

currently in JUNET is one in which the transfer destination is entered

statically for each domain.  This method puts a big load on the domain

master machine of each domain.  During mail transfer using a name

server, since the address of the target machine is found out by

inquiring with the name server and the mail is then transferred

directly to that machine, it is possible to reduce the load on the

domain master and also to make the aintenance operations simpler.

In addition, there is a function by which it is possible to specify

an intermediary machine for mails addressed to domains or machines

that are not connected to an IP (MX record).

The transfer of mails in Internet is already being changed over to

a method using name servers, and hence it is essential to make correct

settings in the name servers if mail has to be received directly

from Internet.

2. Servers and Resolvers

Although normally referred to simply as name server setting, it

actually involves the setting on the server side as well as the

setting on the resolver side (or the client side).  The settings on

the server side consist of preparing the settings file for the name

server (named or in.named in Unix) in order to let the address of

the user's domain known to all, starting the name server, and letting

the presence of that name server known all over the required region.

This type of a name server is called the primary server of that domain.

The files to be set are /etc/named.boot and the files entered in it.

In addition, in order to meet any inquiries coming in while the primary

server is down, it is also very important to start an authorized

secondary server.

A resolver is a group of libraries for calling a name server and

converting the host name into an IP address.  The settings on the

resolver side consist of making the settings necessary for using the

name server from the user's machine.  The files to be set are

/etc/resolv.conf, etc.  Every application (telnet, ftp, rlogin, etc.,)

that needs to get the IP address from the host name will be getting

the IP address by calling the resolver rather than by searching

/etc/hosts.  However, since the actual mechanism involved in this

process differs very widely depending on the machine,

it is necessary to know what type of name servers is the user's

machine compatible with.

Further, very often a name server for caching the data set by others

(identical program as that of a primary server) is also started up

in order not to put any load on other name servers.  Such a name server

is called either an (unauthorized) secondary server or a caching server

(the operations of these two are different).  It is possible to

consider the method of their starting to follow the concept of the

settings on the resolver side and it is possible for a name server to

operate as a primary server for a particular domain while operating

as a secondary server for other domains.

These two settings are independent of each other and it is possible

that only one setting is made while the other setting is not made.

3. Organization of BIND Distributed Servers

The organization of distributed name servers in BIND is described

here.  The intent of having hierarchial domain names is to let the

manager of a domain make the settings within that domain.  In other

words, firstly a root primary server has been determined for the entire

Internet.  The root primary server knows which machines are the name

servers for each of the top domains.  For example, the primary server

at the root of Internet knows which machine is the name server of jp.

Similarly, the server of jp knows the servers of *.jp (such as ac.jp,

go.jp, etc.) and the server of ac.jp knows the machines of *.ac.jp

(such as u-tokyo.ac.jp, keio.ac.jp, etc.).  Therefore, it is possible

for the name server on the searching side to get the address of a

host belonging to any domain by searching down the hierarchy if it

knows at least the root name server.

Therefore, after the setting of the primary server in a domain is made,

it is necessary to contact the manager of the name server of the

higher level domains and get the entries pertaining to the name server

of the lower level domains made in the settings of the higher level

domain name server.

4. Situation Within Japan

Recently, even inside Japan the region having IP links is growing

rapidly.  In addition, the need of making settings of name servers

is increasing also because of the addition of IP links with Internet.

However, it has been pointed out that there is a serious problem

in making the settings of name servers in the domestic network.

This problem is as follows.  When there is a machine that can not be

connected to Internet is present in a name server that can be seen

from the Internet side, and when there is any mail from Internet to

that machine, an attempt will be made to send that mail directly to

that machine.  However, the mail can not be sent because that machine

is not connected to Internet.  On the other hand, it will be very

inconvenient if machines that are not connected to Internet but are

connected to the domestic network cannot be searched using name servers.

In view of this, it was decided to solve this problem using the

following method while operating the IP network within Japan.  At

the time of making the settings on the server side, the name servers

that can be seen from the Internet side and the name servers that can

be seen from the domestic side are started up separately, and machines

that are connected to the domestic network but can not be connected

to Internet are forbidden from being registered in the former name

server.  (The mail may not reach if such machines are registered by

mistake.)  Of course, if the contents registered in these two servers

are the same, (that is, when all the machines can be connected to the

Internet side,) it is possible to use a single server to carry out

the two functions.  Further, it goes without saying that there need be

only a domestic name server for domains that do not have any machines

permitted connection to Internet.

In other words, this implies that there will be two series of name

servers - the name servers in Series A in which are registered only

the machines that are permitted connection to Internet and the name

servers in Series B in which all the machines that are connected to

the domestic network in Japan are registered.  However, since it will

not be possible to search a part of the domestic machines from the

former name server and the foreign name servers can not be searched

from the latter name server, it will be necessary to have a third series

of name servers in Series C from which it is possible to search all

that machines that are connected to both the domestic network and the

foreign network.  Since this series of name servers only inquire the

other two series of name servers, there is no need to carry out any

settings on the server side (that is, these name servers only function

as secondary servers or caching servers) and hence are not a series

in terms of the domain hierarchy.

5. Organization of Name Server Series

While the three series of name servers described in the previous section

need to be prepared, the important thing in this is to clearly determine

which machine actually becomes the server of which series.  The machines

belonging to the different series will be used in the following manner.

Series A: These servers manage the machines that are visible from abroad.

Since these servers are only accessed from abroad, the

reliability will become higher and the traffic too will become

lower if these machines are placed in locations having links

to international circuits.  In addition, the machines belonging

to this series do not use any name servers hierarchially above

themselves but will have to call the name servers of other

machines depending on the setting of /etc/resolv.conf.

Series B: The name servers of this series manage all the machines

connected to the domestic IP network in Japan.  These name

servers can be accessed by the domestic machines that are not

permitted connection to international circuits.  Since these

name servers are not related to Internet, these name servers

should be placed in locations at which they are easily accessed

by domestic network users and, if necessary, it may be better

to start up a secondary server also.

Series C: The servers in this series do not manage any machine and are

accessed by the domestic machines that are permitted connection

to the international network.  Since these servers also access

the international network, they are best placed in locations

having links to the international network and also it may be

better to start up a secondary server.

With the above factors under consideration, the following organization is

being used at present.  (An asterisk * at the end of the server name

indicates that it is a primary server.  All others in series A and B are

authorized secondary servers.)

Series B              Series C              Series A

------------------------------------------------------------------------

root	ccut.cc.u-tokyo(*)    ---		    nic.ddn.mil.(*)

etc.

jp	ccut.cc.u-tokyo(*)    kogwy.cc.keio	    jp-gate.wide.ad.jp.(*)

utsun.s.u-tokyo	    ns.tisn.ad.jp.

*.jp	ccut.cc.u-tokyo(*)    kogwy.cc.u-tokyo	    jp-gate.wide.ad.jp.(*)

utsun.s.u-tokyo	    ns.tisn.ad.jp.

u-tokyo	ccut.cc.u-tokyo(*)    relay.cc.u-tokyo	    ns.tisn.ad.jp.(*)

utsun.s.u-tokyo	    jp-gate.wide.ad.jp.

As has been mentioned earlier, the organization of Series C has

different implications that that of the other two series.  At the time

of starting name servers in each domain, one should start by adding the

entries for that domain in the above table.

Further, when setting new name servers, it is recommended to refer

to the settings made in these machines.

The same method is used for the servers for reverse indexing

(in-addr.arpa).  jp-gate and ns.tisn will function as the secondary

servers for reverse indexing for all the networks connected to

international circuits, and jp-gate and ns.tisn are to be registered

as the servers for Internet.  ccut will be the primary server for

in-addr.arpa which includes only the domestic network.  It will be

possible to search for both the networks connected to Internet and

the networks that are visible only from the domestic side from a

machine in Series C (that is, kogwy and utsun which are at the root)

by making that machine a secondary server of the arpa domain of

Internet and also a secondary server of the domestic in-addr.arpa domain.

This utilizes the fact that the addresses of reverse indexing are

entered directly within the arpa domain in the name servers of Internet.

6. Precautions in Name Server Setting

The precautions that need to be taken at the time of setting name

servers but that have not been described in the reference documents

(1) and (2) are explained below.

6.1 On the 'ANY' entries

In reference document (1), although it has been specified to write ANY

in the information fields not limited to IP, there are cases when the

named field can not handle an ANY entry.  It is preferable to enter IN

in all the fields.

6.2 On the MX record

The MX record is a setting that is very important at the time of sending

a mail.  Although it is convenient to write wild card specifiers in the

MX record, care will have to be taken in doing so.  This is because even

if the MX record has been written using wild card specifiers, the MX

record corresponding to the wild card will not be used if the

corresponding address entry is different.  For example, if the entries

are made as follows:

$ORIGIN			is.s.u-tokyo.ac.jp.

*	IN	MX	5 spica.is.s.u-tokyo.ac.jp.

spica	IN	A	133.11.14.1

acrux	IN	A	133.11.14.5

the mail addressed to acrux.is.s.u-tokyo.ac.jp will not be sent to

spica but will be sent directly to acrux.  When this is to be modified,

it is necessary to specify the MX records individually without using

wild cards as follows:

$ORIGIN			is.s.u-tokyo.ac.jp.

*	IN	MX	5 spica.is.s.u-tokyo.ac.jp.

spica	IN	A	133.11.14.1

acrux	IN	A	133.11.14.5

IN	MX	1 spica.is.s.u-tokyo.ac.jp.

Because of this, it will be necessary to enter the full MX record for

each machine in the case of most machines when although the address is

to be registered but the mail is desired to be received through the

mail server, or when the machine is not connected to IP although it has

been registered in the name server.

6.3 On the machines that cannot receive e-mail

There is a user attempting to send a e-mail to that machine, retrials

will be made continuously until the time out condition occurs

(this may be three days or even seven days).

One method of avoiding this is that of setting another appropriate

machine that can receive e-mail in the MX record of that machine as was

described in Section 6.2 above, and taking the appropriate steps in the

machine that can receive the e-mail in the event of receiving such a

e-mail.  Such appropriate steps can be either determining the name of

the user to whom the e-mail is addressed or sending back the e-mail

to the sender as an error.  In the former case, it is sufficient to

set the name of the machine that cannot receive e-mail as a different

name of the machine that can receive the e-mail by setting sendmail.cf.

6.4 On the MX records for domains

When e-mails are to be received in the form of [user name@domain name],

it is necessary to specify the MX record for that domain.  It is

preferable to specify this in accordance with the JUNET convention so

far.  It is also advisable to use the letter "@" at the time of

specifying the MX records for domains.  For example, the e-mails

addressed to user@is.s.u-tokyo.ac.jp will be delivered to spica if

the specification is made as follows:

$ORIGIN                       is.s.u-tokyo.ac.jp

@          IN        MX       0 spica.is.s.u-tokyo.ac.jp

Apart from this, it is also possible to use the same method when

wishing to receive e-mail to an address that is not a host name.

However, the prerequisite is that such an address should be interpreted

by sendmail.cf as its own address.

6.5 On other records

Since the HINFO record is nothing but a mere comment, whether or not

to set this record can be determined at the discretion of each domain.

It is safer to set the WKS record.  In that case, it is sufficient to

enter only smtp at the present stage (whereby there will be no fear of

losing incoming mail).  The records MB, MR, and MINFO are still in the

experimental stage are not being used frequently at present.

6.6 On forwarders

It is preferable that the name servers of Series C become the secondary

servers of the jp domain.  If they are not, the specification in the

forwarders field should be set for a Series C name server that is at

a higher level than that machine itself and make the corresponding slave

setting.  Here, a name server at a higher level is one that is closer

to the root servers of Series C, that is, closer to kogwy.cc.keio.ac.jp

or utsun.s.u-tokyo.ac.jp.  In addition, in case the higher level server

is down for some reason, it is preferable to specify several servers

in the forwarders field.  In such settings, it is necessary to write

the nearer servers first.

If this is not done, even if the root server is searched once, the

servers jp-gate.wide.ad.jp or ns.tisn.ad.jp will be given as the primary

servers of jp and it will not be possible to get the correct domestic

address.  In addition, this also has the effect of reducing the number of

searches of the name servers for foreign circuits.

6.7 On authorized and unauthorized secondary servers

Although no clear descriptions have been given in the reference

documents, secondary servers are of two types, namely. authorized

secondary servers and unauthorized secondary servers.  An authorized

secondary server is one for which there is an entry of the NS record

in the settings file of the primary server and in the settings file of

the next higher level domain, and cannot be started up without the

permission of the manager of the primary server.  It is preferable

to provide at least one authorized secondary server for each primary

server to take over in the event the primary server is down for some

reason.  In this case, it is not necessary that the secondary server

be placed within the same domain.

The unauthorized secondary servers are the other secondary servers and

can be started up freely.  It is better to conceive of such unauthorized

secondary servers as a part of the settings on the resolver side.

The biggest difference between these two types of servers is that while

an authorized secondary server is accessed when it is not possible

to access the primary server at the time of searching for host names,

etc., from the root side, the unauthorized secondary servers will never

be used unless the address is clearly set in /etc/named.boot or

/etc/resolv.conf.

6.8 Other precautions

The Serial field in the SOA record of the name server settings file

plays a very important role at the time of taking out the data of

the secondary servers.  It is necessary to make sure that the value is

always increased every time the set data is updated.

In addition, when the settings file of a name server is updated,

it is necessary either to start up the name server again or send the

HUP signal. The host names within the settings file of a name server

will be the relative names with respect to the origin of the file

unless there is a period "." at the end of the host name.  Hence,

it is important not to forget entering the final period "." at the

end of an MX record or a PTR record, etc.  The origin of a file is

obtained from /etc/named.boot unless otherwise altered using a

$ORIGIN specification.

It appears that the NS or MX entries should have been completed within

each settings file.  Although this rule is a "suggested practice" in

the case of the name servers of Series A (that is, in the case of the

name servers of domains that can be searched from the root of Internet)

it will apparently (not quite certain) be a "compulsory" rule to be

followed in the case of name servers of Series B with no servers of

Series A.  Particularly, care will have to be taken in the case of

files for reverse indexing in which there is a tendency to forget

this rule.

In the case of the secondary name servers of a large number of domains,

the name server process uses much more memory than is normally thought

and hence care will have to be taken in this regard.  For example,

the name server of utsun is using about 3MB of memory.

6.9 Additional hint

After a name server has been set, very often it is helpful in detecting

errors in setting to dump and check the data using the nslookup command

and to check the dump in the named condition.  It is very helpful to check

the cache file of secondary servers because it is easy to check this file.

7. The Problem of Name Servers being Liars

8. Example of Name Server Placement

8.1 The u-tokyo.ac.jp domain

In the u-tokyo.ac.jp (University of Tokyo) domain, there are both

machines that can be connected and machines that cannot be connected to

from Internet.  Therefore, it is necessary to separately start up the

name servers of both Series A and Series B.  Also, in order to reduce

the traffic and make the searching operations more efficient, it is

preferable to start up a name server of Series C.

In actuality, as is shown in the table in Section 5 above, we decided

to use ns.tisn as the primary server of Series A and ccut as the

primary server of Series B.  In addition, relay is set as the name

server of Series C.  As per the guidelines given in Section 6.6, relay

specifies kogwy and utsun using the forwarders declaration.

Among the client machines in the University of Tokyo, the machines

that can be connected to Internet use relay as their name server.

However, when a large number of client machines are present within a

single organization (such as a division or a department), it is

preferable to have a local name server in order to reduce the load

on relay.  Such a local name server will become an unauthorized name

server of the required domain (that is accessed very frequently) and

specifies relay in the forwarders field.  (Even if it does not become

a full secondary server, it has the effect of improving performance

because this name server carries out caching.)  Although it is possible

to specify kogwy or utsun as the forwarders, it is preferable to

specify relay first in order to reduce the traffic.

On the other hand, the client machines that can not be connected to

Internet use ccut as their name server.  Again, when a large number

of client machines are present within a single organization, it is

preferable to have a local name server.  This local name server will

become an unauthorized name server of the required domain (that is

accessed very frequently) and specifies ccut in the forwarders field.

The forwarders specification is optional.

8.2 The s.u-tokyo.ac.jp domain

All the client machines within the s.u-tokyo.ac.jp domain (Faculty

of Science, University of Tokyo) can be connected to Internet.

Therefore, there is no need to separately start up both a name server

of Series A and a name server of Series B.  In order to reduce the

traffic and make the searching operations more efficient, it is

necessary to start up a name server of Series C.

In actuality, we decided to use ns.tisn as the primary server of

Series A and Series B.  In addition, utsun is used as the name server

of Series C because it is in the local domain.

The client machines within the Faculty of Science use utsun as their

name server.  However, when a large number of client machines are

present within a single organization (such as a department), it is

preferable to have a local name server.  This local name server will

become an unauthorized name server of the required domain (that is

accessed very frequently) and specifies utsun in the forwarders field.

Actually, several secondary servers have been started up so that the

number of machines that directly access utsun as their name server has

been reduced.

9. Correspondence to the Resolver in each Machine and OS

In the following, whether or not named is new indicates whether or not

the function such as directory, forwarders, and slave that are being

supported by the versions 4.8 and later of BIND are present.  Further,

the latest version of BIND is 4.8.3 which is a considerably improved

version of 4.8 and hence it is preferable to use this version.

4.3 BSD

1. A new version of named has been released.

2. All the clients conform to BIND.  However, it is necessary

to make such a setting at the time of preparing the libraries.

Sun OS 4.0

1. The new version of named is the standard.  It is possible

to compile with some minor modifications for BIND 4.8.3.

2. ypserv conforms to BIND.  However, there are some bugs in

the ypserv of Sun OS 4.0.1, but a patching tape is available.

These bugs have been removed in version 4.0.3.  Further,

it is also possible to obtain conformation by replacing the

related libraries among the shared libraries.

In Sun OS 4.0, the descriptions in the manual regarding the method

of making the settings so that ypserv looks at the name server are

hard to understand.  In Sun OS 4.0, it is necessary to mark the

related DBM files for YP so as to refer to the name server.  In

specific terms, by correcting the /var/yp/Makefile

$(MAKEDBM) - $(YPDBDIR)/$(DOM)/hosts.byname;

to the line-

$(MAKEDBM) -b - $(YPDBDIR)/$(DOM)/hosts.byname;

and entering touch /etc/hosts; make, the name server will be referred

to at the time of getting the address from the host name.  In addition,

although by adding -b in the line-

$(MAKEDBM) - $(YPDBDIR)/$(DOM)/hosts.byaddr;

the name server will be referred to even at the time of reverse

indexing, this setting is no recommended because of problems of security.

VAX (Ultrix 3.0)

1. The new version of named is the standard.

2. The clients conform to BIND.  It is possible to set in /etc/svcorder

so that the searching is made in the sequence yp, bind, local.

LUNA (UNIOS-B 1.20Beta)

1. The old version of named is the standard.

2. All the clients conform to BIND.  YP will be searched first

and named will only be searched next if the entry is not found.

Apollo SR10

1. The old version of named is the standard.  It is possible

to compile after making some corrections for BIND 4.8.2.

2. All the clients appear to conform to BIND.  (Apparently,

whether or not named is called is determined depending on

not whether resolv.conf is present but on whether named is

present locally.  However, the details are not known.)

We have not made any investigations regarding the other machines

and operating systems.

10. Distribution

This document can be copied, distributed, and modified freely.

However, when any modifications are made to this document, such

modifications should be clearly described in the subsequent document.

Further, it is forbidden to delete the name of the author or to alter

or delete the contents of the different sections.

In addition, the author does not undertake any responsibility

whatsoever for any damages caused by any discrepancies in this document.

The author would be grateful for communication regarding any errors or

discrepancies in this document detected by any person or persons.

Also, any comments on the contents of this document will be highly

appreciated.

11. Acknowledgments

The author wishes to express his deep gratitude to Mr. Jun Murai,

Mr. Jun Ogami, and many others for their valuable comments on the

contents of this document.

REFERENCES

[1] Dunlap, K.J. and Karels, M. J., Name Server Operation Guide for

BIND (Release 4.8).

[2] manual page for NAMED(8), RESOLVER(3)(5), GETHOSTBYNAME(3).

[3] RFC974, MAIL ROUTING AND THE DOMAIN SYSTEM.

[4] RFC1032, DOMAIN ADMINISTRATORS GUIDE.

[5] RFC1033, DOMAIN ADMINISTRATORS OPERATIONS GUIDE.

[6] RFC1034, DOMAIN NAMES - CONCEPTS AND FACILITIES.

[7] RFC1035, DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION.