CluLink - a binary DX-Cluster/Conference linking protocol

Initial specifications

Heikki Hannikainen

hessu@pspt.fi

===== 0. Definitions ==================================================

Protocol support identifier:  <CLU-1.0>

The identifier tells the connecting node that the node can handle

CluLink protocol version 1.0. The identifier can be placed anywhere

in the login text.

===== 1. Initiating a cluster link: ===================================

Node A                                  Node B

1. Makes connection.

2. Sends login texts,

including the protocol

support identifier, if the

connecting station is allowed

to initiate a link.

3. After identifying the protocol

support identifier, sends the

protocol connection init string.

4. Responds with protocol init

response string, switches to

binary link mode, sends node

and user table, finishes with

message 0.

5. Upon receiving the init response

string, switches to binary link

mode, sends node table and user table,

finishes with message 0.

===== 1.1. Protocol connection init string sent by the connecting node:

CluLink 1.0 nodecall checksum

...where 1.0 is the protocol version to be used, if node B has 1.0

in it's protocol identifier, then it's the maximum number the linking

node should use. If node A only knows version 1.5 and node B gives 2.0,

node a gives 1.5 in init, so that node B knows not to send messages

defined after version 1.5...

Nodecall is the callsign of the connecting node. Maximum 9 characters,

possibly including a SSID (OH7LZB-14 for example).

Checksum is the sum of all bytes in the ctext line containing protocol

identifier, not including the CR. Station B should check that the count

matches, if it doesn't, the connection should be refused. This is just

to make the connection procedure a little more complicated for a guy

trying to mess up in the system... The checksum is an ascii formatted

integer.

Example:

CluLink 1.0 OH7LZB-1 12345

===== 1.2. Protocol connection init response string:

If the link init request is accepted, the node responds with a line

"OK nodecall" and switches to binary link mode.

If the connection is refused, the node responds with a line "NO",

and disconnects after waiting for several seconds, allowing the

response to get trough the network. If a "NO" response is received,

no further attempts to initiate a link should be made.

===== 2. Description of the binary messages: ==========================

Protocol messages are binary, to save valuable network bandwidth.

No CRC checking or message acking is normally done, since this should

be done at lower layers.

First 2 bytes of a message are an unsigned word which indicates the

length of the protocol message, in bytes. This allows message

fragmenting in multiple ax.25 frames. A protocol message can be up

to 16384 bytes long, and the first two bits of the length word are

reserved. They can be used later for indicating, if the message is

compressed, or something else... A protocol message terminates with

a null.

Strings are stored as pascal-type strings. The first byte indicates

the actual length of the string, then follows the string itself.

User and node callsigns are maximum 7 bytes long (6 characters for

the callsign and 1 byte for the SSID). If a null string is to be

transmitted, only one byte of 0 is sent.

Date and time are stored in four bytes, in format used by the rotten

FAT file system. It's accurate to two seconds.

Bits  Description

31-27  hours (0-23)

26-21  minutes

20-16  seconds divided by 2

15-9  year - 1980

8-5  month

4-0  day

The third byte of a message indicates the message type:

--- 0   - 63  ---  Network messages

0  - Link initialization completed

1  - Add node(s)

Entry for one node:

NodeCall: Callsign

Node feature flags: word-sized bitmask: (additions welcome!)

Bit  Meaning

0    Linked trough CluLink (no PC links on the route)

1    CluLink mail implemented

Software type ID: Byte:

0   - unknown

1   - PacketCluster-type (Assumed, if behind a PC link. We don't

actually know about the nodes behind such a link.)

2   - Clusse

3   - DxNet

4   - clx

Version: Word:

High byte: Major version number

Low byte: Minor version number

Link hop count: Byte

2  - Delete node(s)

FromNode: Callsign

- Where the link was lost

Reason: Byte:

0   - Hard disconnect

1   - Node shutdown

2   - Disconnected by system operator

3   - Networking error (for example, looping)

255 - Custom error message follows:

String[30]

NodeCalls: Callsign(s) of the nodes

3  - Add user(s)

FromNode: Callsign

User record:

UserCall: Callsign

User flags: word-sized bitmask:

Bit  Meaning              State:  1                0

0    Here status:                 Here             Away

1    Privileged user:             Privileged       Not privileged

2    System operator:             Operator         Not operator

3    Superuser (sysop, pwd given) Superuser        Not superuser

4  - Delete user(s)

FromNode: Callsign

User callsigns

10 - Change user flags

FromNode: Callsign

FromUser: Callsign

User flags: word-sized bitmask

11 - Change user data

FromNode: Callsign

FromUser: Callsign

Data type: Byte:

1 - Name          (Maximum length 22 characters)

2 - QTH           (80 characters)

3 - Coordinates

4 - Home node callsign (for mail forwarding, 9 chars)

Data string: String

40 - Ping / Traceroute: This message is extended to allow tracing a

route to a node. Can be used when debugging a network or figuring

out how messages are routed. Like the "traceroute" unix utility.

FromNode: Callsign

ToNode:   Callsign

Type:     Byte:

0 - Ping query          } These two are used for a simple ping,

1 - Reply to a ping     } as before. The next ones implement

} the traceroute functionality!

2 - Traceroute query - every time you receive one of these,

you reply immediately with a type 3 message, regardless

of the ToNode field. You still forward the message (unless

it's to you)!

3 - Traceroute reply - indicating "this traceroute query went

trough this node".

Socket:    Byte: Used to identify simultaenous pings/traceroutes

originated from the same node. When replying,

reply with the same socket number.

Hop count: Byte: Incremented every time when sent to a link, ie.

initially 1. Reset to 1 when a replying!

50 - Unspecified "L3" frame:

FromNode: Callsign

ToNode:   Callsign

Unspecified data up to the maximum message length.

This message type can be used to implement software-specific

features, which are not yet supported by any other software.

Just great for experimenting. The message is forwarded

unmodified by all nodes, and only the sender and recipient

nodes need to know what to do with the contents. The node

table can be used to determine, if the recipient is able to

handle a special message type.

--- 64  - 95  ---  DX cluster messages

64 - DX info

FromUser: Callsign

DxFreq: Fixed-point unsigned integer, 4 bytes

- Frequency of the DX station in KHz, multiplied by 10 (eg. one

decimal included). An integer is used to prevent rounding

errors.

DxCall: String, max. 16 bytes

- Callsign of the DX station. Max. 15 bytes long.

Date/Time: Datetime

Info: String, max. 81 bytes

- Additional information string, max. 80 bytes

65 - WWV data

FromUser: Callsign

Date/Time: Datetime

Hour: Byte

SFI: Word

A:   Word

K:   Byte

Forecast: String[80]

66 - Announcement

FromNode: Callsign

FromUser: Callsign

ToNode/Distribution: Callsign:

Any known node callsign: Forward to this node only

'*'  : All nodes

Some other string: Operator-defined distribution. Eg, we would

like to use a distribution 'FIN' which would only include the

Finnish network.

Date/Time: Datetime

Message flags: Byte-sized bitmask:

Bit  Meaning

0    Only shown to system operators

1    Only shown to privileged users and operators

2    This is a weather announcement

Message: String[255]

67 - Talk

FromNode: Callsign

FromUser: Callsign

ToNode: Callsign

ToUser: Callsign

Date/Time: Datetime

Message flags: Byte-sized bitmask

Message: String[255]

--- 95  - 127 ---  Mail forwarding

--- 128 - 159 ---  Conference bridge linking messages

128 - Convers host login (conversd /..HOST)

129 - Set away status (conversd /..AWAY)

130 - Channel message (conversd /..CMSG)

131 - Convers host information (conversd /..DEST)

132 - Invitation (conversd /..INVI)

133 - Set channel mode (conversd /..MODE)

134 - Operator add (conversd /..OPER)

135 - Route query (conversd /..ROUT)

136 - Set channel topic (conversd /..TOPI)

137 - Set user personal information (conversd /..UDAT)

138 - User message (conversd /..UMSG)

139 - Add user (conversd /..USER)

--- 160 - 191 ---  Shared databases

160 - Query a remote database

FromNode,

ToNode

FromUser : Callsign

Socket   : Byte

Database : Byte   (Yes, it's much shorter than a string. When

converting a request coming from a PC link

to the CluLink form, we have to be a bit smart.)

Keyword  : String

161 - Reply to a query

Tonode   : Callsign

Socket   : Byte

Data     : String (can include multiple lines of data in the same

message, separated by CR's. A reply can be divided

to several messages. If the last character is an EOF

(1Ah, ctrl-z), this is the end of the reply.)

162 - Update database

163 - Delete database entry

--- 192 - 255 ---  Miscellaneous messages

255 - Dumb packet

- used for reseting link inactivity timers over a single link.

The content of the message is ignored by the other end.