TN211_11.DOC

NODE PARAMETERS

*******************

TheNet nodes operate on a DYNAMIC ROUTING CONCEPT.   This means the software is

able  to  make  automatic  allowances  for  node  changes  within the  network.

TheNet recognizes  when new nodes are added to the system and deletes old nodes

that have failed.   It does this through periodic broadcasts containing routing

and obsolescence information.

TheNet  nodes  can be also configured to operate  in a stand-alone environment.

There is a  varying  degree of  adjustment  latitude  that  can allow  the node

to  go through  the extremes of recognizing only itself, just its neighbors, or

all other available nodes within the system.

The NodeOp in concert with network mangement has the options of setting  up the

node to operate  "wide open",  or to selectively  include or  exclude  specific

nodes or users.  It is this flexibility that makes TheNet Plus a popular choice

for networking.      One  of  the tools allowing this is a series of parameters

that  establish various node operating conditions.    Most node parameters will

seldom require attention following  initialization.   A few parameters may need

occasional  attention.    The  next five files  goes into detail on each of the

parameters.     The suggested values given for each parameter will allow proper

node operation in a typical system.

Recent versions of TheNet Plus contain two sets of parameters,  those that  are

programmed (fixed) into the EPROM, and those that are settable by SYSOP action.

The over-riding  factor driving  this dual parameter configuration was the lack

of  internal  EPROM  programming space.   With each  parameter  that was fixed,

bytes were saved.   The saved space was used for additional  node features.  In

each  case,  the  candidate  parameters selected for "fixed" set-up, were those

rarely, if ever, requiring change once the node was in operation.