BITTY 1.0

Copyright 1998 by Brian Beezley, K6STI

All Rights Reserved

BITTY 1.0......................................1

Evaluation Version.............................1

Sound Card.....................................1

FSK and PTT Output.............................2

Signal Processing..............................3

Initialization File............................5

Use with RTTY by WF1B..........................6

Use with OH2GI and NA..........................7

Differences from RITTY.........................7

Copyright and License..........................8

---- BITTY 1.0 -------------------------------------------------

BITTY is a high-performance FSK modem that lets you

transmit and receive radioteletype signals on your PC.  It works

in conjunction with a separate terminal program.  BITTY uses

advanced digital signal processing algorithms but does not

require DSP hardware.  It uses your sound card for analog input/

output and optimized assembly language for speed so it can run

right in your PC.  BITTY requires a 486 or better, math

coprocessor, sound card, and DOS 3.1 or later.

---- EVALUATION VERSION ----------------------------------------

The unlicensed evaluation version runs for a limited

time.  For full functionality, put your BITTY.LIC license file

in the directory that contains BITTY.EXE.  (If you rename

BITTY.EXE to XYZ.EXE, rename BITTY.LIC to XYZ.LIC.)  The license

file is valid only for your callsign.  If you plan to use

another callsign for a contest, DXpedition, or special event,

contact K6STI for an additional license file.

---- SOUND CARD ------------------------------------------------

BITTY works with most sound cards, but it works best

with a genuine, 16-bit Creative Labs card.  Other cards may

provide inadequate input/output levels, offer only coarse gain

adjustment, and exhibit limited dynamic range.

BITTY uses the SET BLASTER statement to determine sound-

card hardware settings.  You may need to configure your card to

these settings in AUTOEXEC.BAT by executing a utility.  See your

sound-card manual for details.  BITTY does not require sound-

card device drivers in DOS.

You can obtain receive audio from your transceiver's

speaker output, but it's better to use a fixed-level audio

output when available.  This lets you adjust AF gain for

comfortable listening without affecting BITTY.  Use slow AGC and

set RF gain to maximum.  Connect receive audio to the sound-card

left-or right-channel line input.  If your receiver can't fully

drive the line input, use the mic input instead.

NOTE:  It's easy to overdrive the sound card's high-gain

microphone preamp.  Reducing the input-gain setting won't help

because the preamp comes before the gain-control stage.

Therefore, it's important to check for distortion whenever you

use the mic input.  Tune your receiver past a strong, clean sine

wave.  Look for harmonics generated by an overdriven preamp in

the tuning indicator provided by your terminal program.  You

should see just one bright, steady signal pip as you tune

through the SSB passband.  If your receiver's fixed-level output

overdrives the mic input, add a series resistor (try 3.3k ohms)

to form a voltage divider with the 540-ohm mic input resistance.

1

Type BITTY to obtain a bar graph for input-gain

adjustment.  Use the up- and down-arrow keys to change gain in

2-dB steps.  (Many sound cards have coarser steps and limited

adjustment range.  Some cards may provide no gain adjustment at

all).  The bar graph turns red when the A/D converter clips.

Raise the input gain until clipping occurs, then back it off one

or two steps.  Infrequent, brief clips won't affect copy so

don't worry if the graph occasionally flashes red.  However,

sustained overdrive will cause problems.  Press Esc to quit or T

to set AFSK level.

BITTY can provide AFSK or FSK output.  See the next

section for FSK.  BITTY generates AFSK on the left channel.

Route the left-channel line output (mini phone-plug tip) to your

transceiver's packet, phone-patch, or mic input.  VOX works fine

with AFSK but you can use push-to-talk (PTT) keying if you

prefer.  For PTT you must build the interface circuit described

in the next section.

To set AFSK transmit level, first disconnect your

microphone if active.  Switch your transceiver to LSB and turn

off your speech processor.  Set your mic gain to the position

you normally use for voice (very important).  Set your power-

output control to maximum.  To use VOX, enable it, set anti-VOX

and VOX delay to minimum, and adjust VOX sensitivity so that

your rig keys reliably.  Press T after adjusting input gain.

Use the arrow keys to adjust the output gain for desired output

power.  Once you've initially set output gain this way, it's OK

to make minor power-output adjustments with your mic-gain

control.

Use any text editor to put the input and output gain

values BITTY reports into BITTY.INI.

Carefully check your transmit signal for hum and noise.

If present, add a 10k-ohm resistor in series with the line

output and a 330-ohm resistor across your transceiver's audio

input.  Then reset transmit audio level.

---- FSK AND PTT OUTPUT ----------------------------------------

Some transceivers won't let you use a CW filter in LSB

mode but allow it in FSK.  To permit use of a narrow IF filter,

BITTY can generate FSK output on a serial port (pin 2 for DB25,

pin 3 for DB9).  Use the COMPort parameter in BITTY.INI to

specify the port.  Mark is a negative output voltage and space

is positive.  You may be able to apply the RS-232 signal

directly to your rig's FSK input.  If not, use an interface

circuit like that described next for the PTT signal.  (To invert

FSK polarity, swap the base and emitter leads.)  Ground is pin 7

for DB25 and pin 5 for DB9.

BITTY drives both RTS and DTR (pins 4 and 20 for DB25,

pins 7 and 4 for DB9) of the selected serial port with a PTT

signal.  Transmit is a positive voltage, receive is negative.

2

You can obtain PTT from a parallel port instead (AFSK only).

BITTY drives all eight TTL outputs high for transmit and low for

receive.  For parallel-port output set the COMPort parameter in

BITTY.INI to 378 for LPT1, 278 for LPT2, or 3BC for LPT3.

Connect a 1k-ohm resistor from the RS-232 or TTL output pin to

the base of any NPN transistor.  Ground the emitter and connect

the collector to your PTT line.

---- SIGNAL PROCESSING -----------------------------------------

BITTY's purpose is to copy RTTY signals nearly as well

as RITTY does but with simplified processing.  Here's how it

works:

BPF

BITTY provides a 101-tap, FIR, input bandpass filter.

Use it for QRM when you lack a CW filter.  The BPF is 6 dB down

at 470 Hz, 60 dB down at 920 Hz, and provides more than 100 dB

of ultimate rejection.  Enable the filter with the BPF parameter

in BITTY.INI.  It autotunes along with the channel filters.

Although BITTY's BPF requires half the compute power of RITTY's,

it still may not run on the slowest computers.

Detector

BITTY maintains signal linearity as long as possible.

It doesn't use an input limiter.  This allows it to extract

signals buried in noise and QRM without suffering the limiter

capture effect.

The detector uses a pair of matched filters, one each

for mark and space.  The filters have the SIN(x)/x shape optimal

for the detection of tone bursts in additive, white, Gaussian

noise.  The filter frequencies are 182 Hz apart when fixed.  The

frequencies automatically align to the input-signal tones with

Autotune enabled.

The matched filters feed an I-Q envelope detector.

BITTY does not do post-detection, low-pass filtering.  Any

filtering beyond that provided by the matched filters would

degrade sensitivity.

The matched-filter peaks are quite narrow; tuning is

critical with Autotune disabled.  Try to tune within 10 Hz when

using fixed filters.  Although the peaks are narrow, the filter

skirts drop off slowly.  Strong signals in the skirts can

disrupt copy.  Use the BPF or a 500-Hz CW filter for heavy QRM.

Even without a filter, BITTY's detector is much less sensitive

to QRM than ordinary detectors because it doesn't use an input

limiter.

3

Wide Channel Filters

Although the matched filters maximize sensitivity, they

don't respond well to signals corrupted by severe ionospheric

disturbance.  For example, polar flutter can modulate a signal

with AM and FM components.  The modulation sidebands broaden the

signal spectrum beyond the matched-filter passbands.  BITTY

provides auxiliary channel filters twice as wide as the matched

filters.  Although they degrade sensitivity 3 dB, the wider

filters can improve copy of corrupted signals.  Your terminal

program provides a key to select channel-filter width.

Autotune

BITTY provides automatic channel-filter and BPF tuning.

When enabled the filters quickly lock to the strongest tones

within +/- 250 Hz of center frequency and then slowly track

them.  Autotune accepts shifts between 135 and 250 Hz.

Your terminal program provides a key to freeze the

filter frequencies.  You may need to do this when running a

large pileup to prevent the channel filters from hopping among

the various tones.  The filters are capable of isolating one

signal among many if you carefully tune.

Enable AFSK Autotune from your terminal program when

calling stations.  This automatically aligns the center

frequency of your transmit tones to that of the received signal.

Transmit shift will be 170 Hz for received shifts less than 176

Hz, 200 Hz for receive shifts greater than 191 Hz, and 182 Hz

otherwise.  This feature is possible only when using AFSK.

Receive and transmit shift is 182 Hz with Autotune

disabled.

ATC

BITTY provides automatic threshold correction.  ATC

modifies the threshold that distinguishes mark from space.  The

unmodified threshold is zero (mark and space are equally

weighted).  ATC alters the threshold so that channel-amplitude

differences due to selective fading, IF-filter ripple, and audio

roll-off are compensated.  ATC in effect provides frequency-

diversity reception by automatically shifting the threshold when

a channel fades.

ATC can enable perfect print during selective fades for

signals that otherwise would be garbled.  In addition, ATC lets

you notch out QRM in one channel with your receiver's notch

filter and maintain solid copy using the remaining channel.

BITTY's ATC algorithm peeks into the future to examine upcoming

signal characteristics so it can determine an optimal detection

threshold for the current character.  ATC really helps fight HF

propagation anomalies and interference.

4

---- INITIALIZATION FILE ---------------------------------------

On startup BITTY reads the BITTY.INI initialization file

to obtain a few parameter settings.  Use any text editor to edit

this ASCII file.  Parameter settings use simple phrases with the

parameter name on the left, an equal sign, and the value or

setting on the right.  All parameters are optional; BITTY uses

default settings for unspecified parameters.  Defaults are

indicated by [brackets] in the descriptions that follow.

Source=[L] R M

Specify line-left, line-right, or mic input.  Use the

mic input only when your receiver can't adequately drive a line

input.  Be sure to check for sound-card distortion when using

the mic input.  See the Sound Card section for details.  (Some

sound cards may provide no input-source selection.)

InputGain=[60]

Gain resolution is 2 dB for 16-bit Creative Labs sound

cards.  BITTY rounds down odd-numbered entries.  (Other sound

cards have coarser resolution and limited gain-adjustment range.

Some cards may provide no input-gain adjustment at all.)  The

range is 0 to 80 dB.

OutputGain=[40]

This parameter determines AFSK transmit level.  It has

the same resolution, range, and caveats as the InputGain

parameter.

Output=[AFSK] FSK

When you use FSK you must select a serial port for

output data.  The selected port also provides PTT for both FSK

and AFSK.

COMPort=[0] 1 2 3 4

Select the serial port for FSK and PTT output.  Use 0

for AFSK/VOX.  When the value is greater than 4, BITTY

interprets the number as a hexadecimal port address.  You can

also specify a parallel-port address (AFSK only).

CenterFreq=[2210]

Center frequency is halfway between the mark and space

tones.  Use this parameter to center received tones in the

passband of a CW filter.  The range is 500 to 2500 Hz.

5

UOS=[Off] On

Unshift-on-space automatically reverts the decoder to

LTRS mode whenever a space is received.  This helps prevent

printing long strings of wrong-case text.  However, some modems

don't anticipate use of this feature and emit text streams that

will print incorrectly.

On transmit, BITTY assumes that unshift-on-space may be

used at the receiver.  Therefore, in FIGS mode it always sends

LTRS or FIGS before the first nonspace character following a

space.  This procedure provides compatibility with all decoders.

(LTRS is the Baudot code that causes subsequent characters to be

interpreted as letters.  FIGS shifts the decoder to numbers and

punctuation.)

BPF=[Off] On

Enable the input bandpass filter when you expect QRM.

The filter autotunes along with the channel filters.  It may not

run on slower computers.  There's no need to enable the BPF if

you use a CW filter.

---- USE WITH RTTY BY WF1B -------------------------------------

BITTY works with WF1B's RTTY contest-logging program.

First check all parameters in BITTY.INI.  Settings for contest

use are discussed below.  Type BITTY I to install the program in

memory.  Then start WF1B.  Select the K6STI TNC in the work

sheet.  The tuning-indicator signal pips are within the marker

bars for a properly tuned signal.  After exiting WF1B, type

BITTY U to uninstall BITTY from memory.

You can automate this procedure with a little batch

file.  See CONTEST.BAT for an example.  (CONTEST.BAT invokes the

FONT.EXE utility to provide enhanced WF1B screen fonts.  Type

FONT for more information.)  BITTY sets ERRORLEVEL to 1 when an

error occurs.  You can test ERRORLEVEL in a batch file to take

evasive action.

BITTY requires about 77K bytes of memory.  If you can't

load BITTY and WF1B simultaneously, try loading BITTY in upper

memory with LH BITTY I.

Receiver AGCs often allow peak amplitudes for static

crashes to exceed normal signal levels.  Set the input level to

avoid frequent clipping on the noisiest band you plan to operate

during the contest.

Be sure to set BITTY's center frequency to the center of

your CW-filter passband.  Typical CW filters have poor group-

delay characteristics near their passband edges.  These filters

can distort FSK signals even when their amplitude response is

flat.  Target the filter center to keep signals as far from the

6

passband edges as possible.  If you have a choice between 250-Hz

and 500-Hz filters, use the latter.

Press Alt-T in WF1B to cycle through Autotune off, RX

Autotune, and RX plus TX Autotune (the last state appears for

AFSK only).  BITTY adjusts the tuning-indicator scale so that

autotuned signal pips line up with the markers even for

nonstandard shifts.  Disable TX Autotune when calling CQ so your

transmit frequency doesn't wander.  Press Alt-F to freeze

Autotune (press again to unfreeze).  Freezing Autotune is

particularly useful when running a large pileup.  Disable

Autotune when searching and pouncing with FSK to ensure that

your transmit tones are on frequency.

Press Alt-~ in WF1B to toggle between narrow and wide

channel filters.  The wide filters can improve copy during

severe polar flutter.  WF1B displays "Polar" when the wide

filters are active.  (~ is intended to suggest a wavy signal.

If your keyboard doesn't have this key, use Alt with the key

just below Esc.)

Some modems don't anticipate reception using unshift-on-

space.  After sending a number and a space, these modems don't

resend FIGS before sending another number.  Therefore, contest

exchanges with more than one number (or numerical repeats) may

become garbled.  If the only number in the exchange is RST, it's

probably best to enable unshift-on-space.  (Just remember that

TOO means 599).  If the exchange involves additional numbers,

you may want to disable the feature.  You can always use Alt-L

in WF1B to force the shift state back to LTRS (after the fact).

Better still, when expecting a number WF1B will convert the

letter-equivalent.  Just type or click on what's received.

---- USE WITH OH2GI AND NA -------------------------------------

To use BITTY with the OH2GI Ham System or K8CC's NA

contest logger, follow the WF1B procedure but install using

BITTY X instead of BITTY I.  See the NA manual for control

information.  Use the following keys to control BITTY in OH2GI:

Alt-R   RX Autotune

Alt-A   TX Autotune

Alt-D   Freeze Autotune

Alt-W   Wideband Detector

---- DIFFERENCES FROM RITTY ------------------------------------

Removed:  Standalone mode

Numerical Flywheel

Repetition Integration

Adaptive Channel Combination

Squelch

Ignore stray LF

Add begin/end CR

7

Duplicate LTRS/FIGS

Automatic LTRS/FIGS refresh

Manual LTRS/FIGS refresh

Display both shifts

Show control codes

Word output

TTY output

UMB allocation

PACTOR

Replaced: Wideband detector with wide channel filters

Altered:  BPF bandwidth         470 Hz

Fixed:    Baud                  45

Nonautotune shift     182 Hz

ATC                   Auto

Autotune speed        M

Autotune width        500 Hz

Stop bits             1.4

New line              CR LTRS

Punctuation           TTY

Display case          Upper

---- COPYRIGHT AND LICENSE -------------------------------------

This software is copyrighted.  It's licensed, not sold.

When you purchase a license for a software product, you're

granted the right to use the software under certain conditions;

ownership of the software remains with the copyright holder.

Licensing software is different than purchasing a tangible

object that you may use or dispose of as you wish.

This software is licensed to individuals at discount for

amateur use.  It has been provided to you on the condition that

you abide by the following:

1.  You will not use the software for professional, business,

government, military, or institutional purposes.

2.  You will not sell, rent, lend, give away, or otherwise

transfer your BITTY.LIC license file to others.

Brian Beezley, K6STI       (760) 599-4962

3532 Linda Vista Dr.        0700-1800 PT

San Marcos, CA 92069        k6sti@n2.net

8