BPQSPI

The Serial Port Telephone Interface for G8BPQ network nodes.

Written by Larry Houbre, AA1FS

========

Overview

========

BPQSPI allows you, the G8BPQ node sysop, and your users access to your

network node via a telephone modem. Once connected to BPQSPI, after a

simple log-in process, the user is connected to the node and may then

invoke any of the node commands, connect to any of the nodes on the node

list, or to the accompanying BBS. Quite simply, using BPQSPI is

identical to being on packet...but over the phone!

==============================

Program and accompanying files

==============================

The following files are standard with the program, and will come

archived in BSPI***.LZH or BSPI***.ZIP, where *** is the numerical

version of the software.

FILENAME      DESCRIPTION

------------  ---------------------------------------------------

BPQSPI.EXE    The executable file, which runs as a TSR (terminate

and stay resident) program.

BPQSPI.USR    An example of the user file.

BPQSPI.DOC    This documentation.

The following optional files can be added to enhance operation of your

BPQSPI system. They ARE NOT included in the archived file.

FILENAME      DESCRIPTION

------------  ---------------------------------------------------

BPQSPI.HLP    Offers a brief helpfile for users working at the

BPQSPI prompt. Should be no more than 300 characters.

BPQSPI.HI     Used to create a brief log in message. Should be no

more than 300 characters.

BPQSPI.MI1    A set of actual modem commands to be sent to the

modem at start up.

BPQSPI.MI2    A set of actual modem commands to be sent to the

modem between callers.

=============

BPQSPI setup.

=============

Initial setup of BPQSPI is very simple and is completed in four steps.

Step 1 - Installation

---------------------

Unarchive the program, and copy its files into your BPQ directory. All

pertinent files should be in this directory to ensure proper operation

of BPQSPI.

Step 2 - Modification of AUTOEXEC.BAT file.

-------------------------------------------

Add the following line to your AUTOEXEC.BAT file. This line must be

inserted BEFORE any of the G8BPQ files are loaded! Ideally, insert

this line just above your BPQ files.

BPQSPI switchname port baud

Where Switchname = Node Switch's ALIAS, i.e. WHALE

Where Port       = Com port, 1-8

Where Baud       = Baud rate, 300 to 115200

Example: BPQSPI WHALE 1 19200

The above example shows the node alias as WHALE. The modem is connected

to the computer's com port 1, and the modem's speed is 19,200 baud.

Provided the above info is correct and the line is added BEFORE any of

the other BPQ files in AUTOEXEC.BAT, the program will load, terminate,

and stay resident, operating in the background.

Note: If using non-standard port addresses or IRQs, you can and must

specific the port address and IRQ as $h,i. For example, port address

$3F8 and IRQ 5 would be   BPQSPI WHALE $3F8,5 19200

Step 3 - Modification of BPQCFG.TXT file.

-----------------------------------------

BPQSPI fools G8BPQ into thinking it is talking to a KISS TNC, when of

course it is actually talking to a modem. A port must be added to

BPQCFG.TXT. Follow the forthcoming example as a guideline, changing your

IOADDR and SPEED as necessary.

PORT

ID=MODEM

TYPE=EXTERNAL

PROTOCOL=KISS

IOADDR=3F8H     (3F8H=com1, 2F8H=com2, 3E8H=com3, 2E8H=com4)

INTLEVEL=120    (absolutely DO NOT change this!)

SPEED=19200     (computer to modem speed)

* CHANNEL=A

QUALITY=0       (no need for nodes broadcasts to a modem!)

MAXFRAME=3

* TXDELAY=35

* SLOTTIME=100

* PERSIST=100

* FULLDUP=1

* SOFTDCD=1

* FRACK=1000

* RESPTIME=2000

* RETRIES=3

* PACLEN=225

ENDPORT

The values of the above parameters marked with astericks are irrelevent,

however they MUST be in the port configuration for BPQSPI to function!

Remember, BPQ thinks this is a KISS TNC it's talking to. You'll also

likely want to add a little blurb in BPQ's info file section reflecting

the addition of the phone port. Remember to run BPQCFG.EXE to compile

the changes to BPQCFG.TXT.

Step 4 - Create/edit the BPQSPI.USR file.

-----------------------------------------

BPQSPI.USR can be created or edited using any ascii editor, such as

DOS 5 or 6 EDIT, QEDIT, and others. Follow the format in the example

below.

AA1FS TEST

KC1UA BETATEST

WA1YKN NETWORK

Use the actual callsign only, DO NOT use packet radio SSID's (KC1UA-1

won't work). Users MUST have a password in order to connect to the node.

The BPQSPI.USR file may be edited in real time; there is no need to

reboot the system after adding a new call and password to the file.

Optional Step - Modem Init File(s)

----------------------------------

At start up, BPQSPI will send the following commands to the modem:

ATZ

ATV0

ATE0

ATQ1

ATM0

ATS0=1

ATS7=60

If you wish to change some modem parameters, you can create a file named

BPQSPI.MI1, which will simply be sent to the modem AFTER the above commands.

The line in the file should be a complete modem command WITH the AT prefix.

In addition, after BPQSPI hangs up, it will send any commands found in the

file BPQSPI.MI2 if it exists. This file should have as few commands as

possible to reset the modem between calls.

=====

Your BPQSPI setup is now complete! Provided the above was done properly,

a reboot of your system should initialize your modem, put it in auto

answer mode, and render BPQSPI ready to take your users' phone calls.

============

Using BPQSPI

============

1. Dial the BPQSPI number using your favorite communications software.

2. You will receive the CONNECT SPEED (i.e. CONNECT 14400) message.

3. Wait for the BPQSPI *.** prompt (where *.** is the software version)

to appear on screen. BE PATIENT! Some systems seem to take up to 45-50

seconds before this prompt appears. This is something that will be

addressed in future versions of the software. If the sysop has

created the BPQSPI.HI file, you will also see a brief log on message.

4. Type L, followed by your callsign and password, to log in. (i.e.

L KC1UA BETATEST). This may be in either upper or lower case letters.

You will then see a message indicating you're logged in.

5. You are now at the BPQSPI prompt. Typing a question mark and hitting

enter will provide the following help information:

BPQSPI Commands:

B           - Quit BPQSPI (disconnects modem)

C           - Connect to Switch (connect to accompanying node)

ECHO on/off - Changes echo of characters you type (normally ECHO ON)

L call pass - Log in (initial login before connect to node)

LF on/off   - Changes status of adding linefeeds (normally LF ON)

S           - Status (shows settings and connect status)

SUI         - Show UI frames (allows monitor of certain packets)

?           - This help

HELP        - Additional help file created by sysop (BPQSPI.HLP)

6. To log onto the node, use the C command. You will then be connected

to the accompanying network node, and may then invoke any commands as

you normally would on packet radio. (HINT: When you connect from the

accompanying node to another node or BBS, use the S option at the end

of the connect string. An example of this is C SWL S. This will

connect you to the SWL node, but when you disconnect from it, you

will be returned to BPQSPI's accompanying node. If you do not use the

S option, you will be returned to the BPQSPI prompt and will have to

log back into the system again from there.)

7. To log off of the system, tell the node BYE in the usual manner by

typing B and hitting enter.

8. At this point, you are returned to the BPQSPI system, where you can

invoke any of the above commands. Another B logs you off of the

system. If you wish to access the node once again, you will need to

log back on with your callsign and password.

==========================

Known problems for sysops.

==========================

1. Initial delay in sending BPQSPI prompt. Seems to be random...some

modems display prompt immediately, while others can take up to 50

seconds or so before user is able to log in.

=========================

Known problems for users.

=========================

1. With the exception of the initial log on display experienced with

some systems, BPQSPI is essentially painless and simple for all users

to learn and operate.

=======================

Additional information.

=======================

For sysops running AA4RE BBS in addition to BPQ: W1NPR has written a

utility program, BBUCV3A.ZIP, that will convert AA4RE phone port users

callsigns and passwords to the format used in BPQSPI.USR. Sure beats the

pants off of doing it manually!

BPQSPI.EXE can be loaded high, provided you have a big enough upper

memory block. BPQSPI uses about 85K, so it appears. If you wish to do

so, ensure that you have the statement DOS=HIGH,UMB in your CONFIG.SYS

file.

Sysops using DESQVIEW should ensure that BPQSPI is loaded before

DesQView, and absolutely before any other BPQ files.

BPQSPI is free for use by Amateur Radio Operators in Amateur

Radio Communications applications. Any other use is prohibited.

Please direct all comments to Scott/KC1UA@KC1UA.#EMA.MA.USA.NA. I will

ensure that they are discussed/forwarded to Larry immediately. (Of course,

a direct word of praise to Larry/AA1FS@AA1FS.#EMA.MA.USA.NA doesn't hurt.)

If you would like to see BPQSPI in operation, drop a packet note to the

above address and I will be glad to set you up with a password.

Messages can also be left the the landline BBS, Dreamer's BBS, at

508-991-6058 ... fidonet 1:329/863.

73 and enjoy! Scott/KC1UA for Larry/AA1FS.