W P S  ( W i s p  P o w e r  S a v e r )

Software and hardware for power saving during unattended

satellite opertation with the WISP package.

(Free for Radio Amateurs, No Commercial Usage)

Walter Crauwels, ON4BCB (ex ON1AOT)

04/01/97

Introduction:

------------

As many Wisp users, unattendent satellite earth stations have always theire

PC,tranceiver,powersupply,rotor controller,TNC,PA,etc.. powered up, 24h/24h,

even if there is no satellite above the horizon for hours.

This costs you a lot of electricity over the year,and this is absolutly no

good for your equipment. (i think we all agree !!!)

Therefor i designed a solution called WPS (Wisp Power Saver (C) )

WPS is a hardware circuit using a microcontroller which will switch off

all your equipment during the time that there is no satellite above

the horizon.

As soon as there is a satellite, WPS will wake up your complete hardware

system (PC,tranceiver,powersupply,rotor controller,TNC,PA,etc..) a few

minutes before AOS.

After LOS of the satellite it will shut down (sleep) your system after a user

determinated time and wake's it up again before the next AOS.

You will have to specify in WPS.INI if you are using GSC 16 or 32 bit.

If WPS becomes popular we can ask Chris to implement the WPS.EXE directly

into future versions of GSC.

So you only will need the hardware.

So as we all know Chris is verry busy,so let's write some mail to Chris

if you enjoy WPS.

Copyright

---------

WPS hard and software is copyright protected.

WPS may be passed freely to others for the usage in Amateur Radio.

It is not permitted for the Program to be used commercially

or to be sold commercially.

A guarantee for a correct functioning will not be given. The User

has no right of recourse, in the event of a Malfunction of WPS and/or

their connected equipment to WPS.

Quick Start

-----------

- If you receive WPS.ZIP it will contain the following files:

WPS.DOC		This documentation file

WPS.INI		The initialisation file

WPS.EXE		The .EXE file

WPS_SCH.HPG		WPS hardware schematic in HPGL format

WPS_SCH.PCX		WPS hardware schematic in PCX format

WPS_SCH.JPG		WPS hardware schematic in JPG format

WPS_PCB.HPG		PCB layout in HGPL format

WPS_PCB.PCX		PCB layout in PCX format

WPS_PCB.JPG		PCB layout in JPG format

WPS_SLK.HPG		PCB layout in HPG format

WPS_SLK.PCX		PCB layout in PCX format

WPS_SLK.JPG		PCB layout in JPG format

WPS.BOM		Parts list

HISTORY.TXT		Version history

MSAFINX.DLL		Visual Basic DLL files

VBDB300.DLL

VBRUN300.DLL

THREED.VBX		Visual Basic VBX files

MSCOMM.VBX

Hardware setup and testing:

- Assemble the PCB and power it up (a green led will flash 3 times)

- Connect a RS232 FULL wired DB9 cable to the WPS hardware or make a cable:

DB25: (PC)	DB9: (PC)	DB9 (WPS hardware)

Pin: 2		3		3

7		5		5

8-20-6		1-4-6		-

5-4		7-8		-

- Use a small RS-232 Terminal program and set it for:

- 9600bd,8bits,no parity,1 stopbit,

- Comport where WPS is connected to.

Send in capitals: WU@0001 <return>

Look directly to your WPS led, it will flash 2 times if it receives

the correct string,then it will wait 45sec,the led wil become RED,and

will become green again after 1 minute.

If you passed these tests, your hardware is working properly.

Setting up the software:

* GSC 16bit under WIN95 or WIN3.11 *

- Copy the following files to the \WINDOWS\SYSTEM directory.

- MSAFINX.DLL

- VBDB300.DLL

- VBRUN300.DLL

- THREED.VBX

- MSCOMM.VBX

- Copy the following files to the C:\WPS directory. (This MUST BE C:\WPS

due to a problem in Shedule events from GSC16bit)

- WPS.EXE

- WPS.DOC

- WPS.INI

- edit WPS.ini and set:

- your comport number and the path where Wisp.skd will be found

(the file name wisp.skd will set WPS for GSC 16bit)

- set the SleepAfterLos time (This is the minimum time when WPS will shutdown

if no satellites are visible (Default: 180 Min)

- set the WakeUpBefore AOS time (Time before pass to wake up your system)

(Default 10 Min)

Example for GSC 16bit:

[global]

FileName=c:\wisp\wisp.skd	'Name and path of the shedule file(wisp.skd = 16bit GSC,wispsc~1.dat = 32bit GSC)

TZ=60				'Difference in minutes between UTC and local Time

SleepAfterLOS=180		'in minutes (default 180 Min)

WakeupBeforeAOS=10		'in minutes (default 10 Min)

CommPort=2			'1 = com1; 2 = com2 ...

Comm=9600,N,8,1			'9600 bps;no parity;8 databits;1 stopbit (do not change)

ExitWindows=1			'Exit Windows 0/1 (default 1)

- add the following to your autoexec.bat as last lines:

set TZ = LOC-1

WIN

- RUN  WPS.exe after every pass, setup a Schedule event in GSC

Name: Wisp Power Saver

Program C:\WPS\WPS.EXE

Time: 2 Min

Run: After Pass

Associate: All Satellites

Satellite: None

Activate: Yes

Run Minimised: NO

- Connect a Relay to the open collector output from your WPS hardware

This relay will switch your equipment on or off

- Quit GSC and start up again

* GSC 32bit under WIN95 or WINNT *

- Copy the following files to the \WINDOWS\SYSTEM directory.

- MSAFINX.DLL

- VBDB300.DLL

- VBRUN300.DLL

- THREED.VBX

- MSCOMM.VBX

- Copy the following files to the C:\WPS directory. (This MUST BE C:\WPS

due to a problem in Shedule events from GSC 16bit)

- WPS.EXE

- WPS.DOC

- WPS.INI

- edit WPS.ini and set:

- your comport number

- the path where wispsc~1.dat will be found

(the file name wispsc~1.dat will set WPS for GSC 32bit)

- set the SleepAfterLos time (This is the minimum time when WPS will shutdown

if no satellites are visible (Default: 180 Min)

- set the WakeUpBefore AOS time (Time before pass to wake up your system)

(Default 10 Min)

Example for GSC 32bit:

[global]

FileName=c:\wisp\wispsc~1.dat	'Name and path of the shedule file(wisp.skd = 16bit GSC,wispsc~1.dat = 32bit GSC)

TZ=60				'Difference in minutes between UTC and local Time

SleepAfterLOS=180		'in minutes (default 180 Min)

WakeupBeforeAOS=10		'in minutes (default 10 Min)

CommPort=2			'1 = com1; 2 = com2 ...

Comm=9600,N,8,1			'9600 bps;no parity;8 databits;1 stopbit (do not change)

ExitWindows=1			'Exit Windows 0/1 (default 1)

- add the following to your autoexec.bat as last lines:

set TZ = LOC-1

WIN

- RUN  WPS.exe after every pass, setup a Schedule event in GSC

Name: Wisp Power Saver

Program WPS.EXE

Program path: C:\WPS

Time: 2 Min

Run: After Pass

Associate: All Satellites

Satellite: None

Activate: Yes

Run Minimised: NO

- Connect a Relay to the open collector output from your WPS hardware

This relay will switch your equipment on or off

- Quit GSC and start up again

Principle of Operation

----------------------

As your PC boots, Windows will be started, GSC (16bit) will be running.

After the first PASS WPS.EXE is called, it will check if there is a new

pass within "Sleep after LOS" specified in WPS.ini

ex: Current time is 20:00 , Sleep after LOS is 120 minutes

If there is a pass within the next 120 minutes WPS will close and do nothing

If there is no pass within the next 120 minutes WPS will send a time string

to your WPS hardware.

Then it will close all your open windows and exit windows.

BE CAREFULL that there are NO DOS WINDOWS open, it is not possible to

close DOS applications automaticly !!!

ex: Current time is 20:00 , Sleep after LOS is 120 minutes, Wake up before AOS

is 10 minutes, next pass will be at 23:45.

WPS time string will be: 23:45 - 20:00 - 10 min = 215 min

As soon as WPS hardware receives the time string, it will wait 45 sec

and will power down your complete system.

AT 23:35 (23:45 - 10 min) WPS will wake up your complete system,

Your PC will boot and start all over.

WPS hardware:

------------

The WPS hardware consists of a Microcontroller which handles the time

string sended by WPS.EXE.

It is connected to a serial Port,trough a DB9 FULLY wired cable.

It has a Bi-color led which shows the status (Sleep/Wakeup).

The output is an open-collector type,so that you can connect a Relay

or solide state relay to switch the mains of your equipment.

WPS has a pushbutton to wakeup your equipment if it is sleeping.

Where can i Get WPS ?

-------------------

WPS is NOT available as a kit, schematic's and PCB layout are included

in the distribution file.

You can only obtain a programmed micro controller by me.

The price of the programmed microcontroller will be 10 US dollar + shipping

Shipping: Benelux: 2 Dollar (50 BFR)

Europe:  3 Dollar (100 BFR)

World:   5 Dollar (150 BFR)

I only accept cash money in US Dollar or Belgian Franks

Anyone who has problems to get the other parts, should send me a message

and i will see what i can do.

Walter Crauwels ON4BCB

Waversesteenweg 210

B-2500 Lier

Belgium

AX25:	ON4BCB @ ON1ANR-5

Email:	ON4BCB@mail.dma.be

URL:	www.bewoner.dma.be/ON4BCB

Please particularly observe the foregoing Text!  I make no financial

gain, it is not possible for me to pay the postage and other costs out of

my own pocket.

A special thank goes to:

Guy Roelant ON1BHM for writing my idea into an EXE file

Erwin Wuyts ON7VN for future beta testing under WIN95 32bit

Frank Saudemont ON4CCF for beta testing the soft and hardware under WIN95

Julien Goethals ON6GJ for beta testing the soft and hardware under W3.11

Willy Wittesaele ON1AWU for the first tests with the microcontroller

Dirk Collone ON1AEI for the first software version

Warning: !!!

------------

IT is NOT good to switch electronic devices ON and OFF several times a day

So do NOT set "Sleep After LOS" to fast.

The idea of WPS was to switch off your system during the long periodes

without passes (5 and 10 hours).

So normally WPS will shutdown once a day, which is no problem, as we

manually switch on our system 1 or 2 times a day.

73s from Walter, ON4BCB @ ON1ANR-5